RR9000 Demo program user guide

Testing Environments

Support 15693 series reader/writer under Windows 98 , 2000 & XP operation system.

Basic operation

  - Make sure the reader is installed propertly before run the demo program

  - run the demo program found in the CD

- CD directory \15693_demo_program\ RR9000_demo_eng.exe

  - select from menu , click on “open COMPort” to establish the connection automatically, reader information will show as Fig.1 area “8”

  - place the Tagi-it ISO card on the reader

  - click on  “Inventory” button , the ID number will show on fig.1 area “19”

  - click on the down arrow on the operation mode to select the ID and click on the “getsysteminformation”,the tag information will show on fig 1 area “10”.

Tag-it memory config : writing type “A” , max block : “0-63” , block size “4”

  - click into the memory organization table to select a block to be read/written to

  - select the block no (0-63) then read or write the data from/to the tag

Remake :

After inventory process, the identified tags are switched to a Quite state.

Therefore , the RF field is reset automatically before each inventory process so that the tags are identified continuously.

Below are two methods to send the “Inventory” command for the same Tag .

[1] click on “close RF” “open RF” ”Inventory” , before you click on the “Inventory” command for the same tag

[2] take the tag away from the RF field , then place it on the reader again so that the reader can active the tag again. Then Click on “Inventroy”

Fig 1 – demo program menu.

- communication : open/close com port

- Power management : switch the RF field off /on

- change reader address : hex address 00-FE

- Output state control (for 15693-RW-232-1W & 15693-RW-232-4W version only)

- Input state control (for 15693-RW-232-1W & 15693-RW-232-4W version only)

- Relay state control (for 15693-RW-232-1W & 15693-RW-232-4W version only)

- Get antenna status (for 15693-RW-232-4W multi antenna version only)

- reader information : show reader information after Reader connected with PC correctly

- operation mode : address mode , selected mode & non-address mode (refer to Tag-it datasheet)

- Tag information : show tag information after click on the “getsysteminformation”

- Block operation : click into the memory organization table to select a block to be read / written to (Tag-it memory–Tag writing type:A/Max block:0-63:Block size :4

- to 14.  other implemented Tag-it commands as follow can be used according to their function defined in datasheet : reset to ready / stay quiet / Quite storage

: AFI (application family Identifier) read/write function

: DSFID (data storage format Identifier) read/write function

15. set active antenna : (for 15693-RW-232-4W multi antenna version only)

16. max Inventory scan Time : set the scan time for inventory command

17. Inventory : read DSFID-UID number

18. Inventory Multiple tags : (for 15693-RW-232-1W & 15693-RW-232-4W version only)

19. Inventory DSFID-UID list : message window to show the DSFID-UID data

20. Test mode : (for 15693-RW-232-1W & 15693-RW-232-4W version only)

Press “start” to read multi tags at the same time as follow menu .

Test Mode Menu